# OPC UA Client — Setup & Run Instructions

# 1. Purpose of the Project

This document explains how to install the OPC UA simulator, configure dummy tags,
run the Python OPC UA client, and generate hourly data logging sheets.

# 2. Software Components Used

• Unified Automation ANSI‑C Demo OPC UA Server (Simulator)
• UAExpert Client (for browsing tag nodes)
• Python 3.x with opcua & pandas libraries
• VS Code / Command Prompt for execution

# 3. OPC UA Simulator Setup

Step‑1: Install the Simulator
1) Download and install Unified Automation ANSI‑C Demo Server
2) Launch the application: UaAnsiCServer.exe
3) Allow Windows firewall access if prompted

Step‑2: Confirm Server Endpoint
The running server displays an endpoint similar to:

opc.tcp://LAPTOP‑FRV8HPM7:48020

This URL is used by the Python client to connect to the simulator.

# 4. Browsing & Selecting Dummy Tags (UAExpert)

1) Open UAExpert
2) Add a Custom Discovery connection using the above endpoint
3) Trust certificates if prompted

Navigate to:

Objects → Demo → 000_Static → Scalar

10 dummy tags were selected from this list (Int32, Int16, Integer, Number,
NodeId, ImagePNG, HeaterStatus, Guid, Float, Double).

Their Node IDs were copied and mapped into the Python client script.

# 5. Python Client Setup

Install required Python modules (first time only):

pip install opcua pandas

Ensure the simulator is running before executing the script.

# 6. Running the OPC UA Client

1) Open the project folder
2) Run the script:

python opc_logger.py

The client connects to the OPC UA server and reads 10 tags every 60 seconds.
For each read cycle, a row is written to a CSV log file.

# 7. Hourly Logging Behaviour

The script generates log files in the format:

OPC_Log_YYYY‑MM‑DD_HH.csv

Each file contains:
• Timestamp (24‑hour format)
• Epoch timestamp (UTC)
• 10 dummy tag values

When the hour changes, the client automatically switches to a new CSV file.
The program runs continuously until manually stopped.

# 8. Output Verification

To verify correct execution:
• A CSV file should be created for each hour of runtime
• Rows should appear every 1 minute
• Columns must match assignment specification

At least two hourly log files are included as sample output.

# 9. Stopping the Application

To stop the logger, press CTRL + C in the terminal window.
Restart by running the script again if required.

# 10. Notes & Assumptions

• Anonymous authentication and unsecured endpoint were used for simulation
• Values are dummy data for training/assignment purposes
• The script may be extended to subscriptions or secure endpoints if required

# 11. Conclusion

The system successfully demonstrates OPC UA client‑server interaction,
periodic data acquisition, and automated hourly logging suitable for industrial
data collection and analysis workflows.